### Installing a connection to an ABB database

You need the following items and information to install a connection to the ABB database.

- Microsoft ADO installer with Microsoft ODBC for Oracle driver (mdac_typ.exe v2.7 or later). This is available free from the www.microsoft.com site

- Oracle Client for Windows in the version corresponding to the ABB MPS Oracle version (e.g. v8.0.5 – 10.2.0.1). The latest Oracle client packages are included on the InkCenter installation CD.

- IP addresses / names of the InkCenter platform and the ABB database server.

Perform the following steps to establish and test the database link.

- Ensure that the InkCenter PC and the ABB database server have a network path available, and that you can ping between the machines. As they are likely to be on different sub-nets then you may have to define gateways or add routes to establish this connection. For the PC end this will typically be a command like “ROUTE –p ADD ABB_IPsubnet Mask 255.255.255.0 InkCenterPC_IPAddress”.

- Add an entry to the PC Hosts table for the ABB machine (e.g. 192.168.1.1 MPS700DB). You should now be able to ping by name.

- Install the Microsoft ADO by running the Mdac_typ.exe installer (if not already installed).

- Install the Oracle Windows client support by inserting the Oracle CD. This will auto-run.

- Start ODBC Administrator and create a new System DSN

- Click Yes if a warning dialog appears.

- Select Add New Service with a Service name of MPS700DB. Next

- Select TCP/IP. Next

- Enter Host name MPS700DB and Port 1524. Next

- Enter Database SID of MPS. Next

- You can test the service by pressing the test button.

- Click Finish.

- You can further test the Oracle set up by entering TNSPing80 MPS700DB 3 on a command line. You should get 3 Oks with a time response.

- Configure an ODBC System DSN

- Start up the ODBC control panel applet.

- Select the System DSN Tab

- Select Add and choose the Microsft ODBC for Oracle driver. Click Finish

- Enter MPS700DB into both the Data Source Name and Server fields. OK and Close.